JS series Digital Display Timing Meter

1.Overview

JS series digital display timing meter has the merits of displaying accurately and clearly, digital dialer setting conveniently, high control accuracy, large range of its delay time, excellent anti-disturbance capability, complete specification, etc. It can be used widely in metallurgy, machinery, power supply, chemical engineering, light industry, mine, medical treatment and so forth, to act as time delay, time display and control on timing.

2. Technical Characteristics

2.1 Mounting kind: panel insert kind or socket mounting kind.

2.2 Contact capability: AC220V, 3A ( resistance load)

2.3 Count life time: ≥10 ten thousands times.

2.4 Deviation in the time counting: ≤±0.1%

2.5 Ability in anti-disturbance: 3 class (IEC-801)

2.6 Power supply: 85~110% of the rating power supply voltage

2.7 Power supply voltage: alternative current 12V, 24V, 36V, 110V, 220V, 360V/50Hz, direct

current 12V, 24V.

2.8 Power consumption: ≤4.5W

2.9 Operating environment: temperature -5℃~40℃, relative humidity≤±90%

2.10 Accuracy class: A class:≤0.3%±0.05s, class C: ≤1.5%±0.05s

class E: ±0.05s in sync with the power supply

2.11 Range of time delay: showed in the table as follows, s-seconds, m- minutes, h-hours

3. Specifications

Notes:

(1) When order, the user ought to determine the model and specification according to

model series of the meter. As an example, if the model is JS11, the range of delay

time is1s~99m，two sets of "turn on" with time delay and one set of momentary output

are needed and the power supply is 220VAC/40Hz, then the model specification is

JS11S-1A21H-220VAC.

(2) JS14S and JS48S have two kinds of terminals: 8pins(8P) and 11pens(P). If user does not

give clear its indication, JS14S will be with 11pins and JS48S with 8pens.

(3) The range of dual setting T1/T2:: 9.9s/9.9s, 99s/99s, 99m/99m, 99s/9.9s, 9.9s/99m, 99h/99m

99m/9.9m, 99h/9.9h, 9.9h/99h.