JY-Z Series Electronic Accumulating Counter

1. Overview

JY-Z series digital display electronic counter adopts the micro-computer with excellent performance as its main control component,. has many merits, such as high accuracy, large range of number counting, digital display, permanent retention time for power cut, strong capability in anti-disturbance, pretty appearance, etc.

JY-Z series electronic counter can be used widely in the industries of packing, printing, pharmacy, food, textile, papermaking, petroleum, chemical engineering, metallurgy, etc, operating as counting and display. It can also display and control the rotating revolution, length and other parameters.

2. Features

2.1 Mounting kind: panel mounting kind or socket mounting kind

2.2 Maximum counting frequency:

High speed (H): 5000times/s,   Model speed (┏┓): 250times/s

Low speed (L): 25times/s  ( the frequency signal ratio are 1:1 for above three speeds.)

2.3 Input signals:

(1) Contact signal: relay, distance switch, microswitch, etc

(2) Non-contact signal: approach switch, photoelectric switch, etc.

(3) Pulse signal: TTL circuit and so forth

Note: CP1 is the input terminal of negative pulse counting (NPN) signal and CP2 is the input terminal of positive pulse counting (PNP) signal. The pulse voltage: low level VL=0~1.3V, high level VH=4.5~30V).

level VL=0~1.3V, high level VH=3.8~30V).

2.4 Anti-disturbance intensity: 3 class (IEC-801)

2.5 The input impedance of signal: 5 kΩ

2.6 Data retention time for paramaters and data when the power is turned off: ＞10 years

2.7 Reset (clear to zero) kind: when reset, the values of display and control output all clear to zero

(1) Reset with the reset key on surface panel

(2) Reset with terminals (make short of the pins of RST and COM)

2.9 Power supply: refer to model series.

2.10 Output contact capability: AC220V, 3A (for resistance load)

2.11 Power consumption: ＜4.5W

2.12 Operating environment: temperature 0~50℃，relative humidity≤95%

3. Model Series

4. Specifications

Notes:

1. In above table only the ordinary specifications are listed, the special specification can be designed and manufactured by user's request.

2. For the JY202 socket model instrument, its length is 106mm, inserted depth is 100mm, and for the terminals instrument, its length is 90mm, insert depth is 84mm.